LAWS OF MOTION: TENSION PROBLEMS I

Name: _______________________________________________Period: ______ Date: ________

1. Two blocks, m1= 6kg and m2= 4 kg blocks are hanging from a frictionless pulley and released at rest. Find:

a. The acceleration of the system. _____________

b. The tension in the string. _____________

2.  A mass on a frictionless table is attached to a hanging mass over a frictionless pulley as shown. Find:

a. the acceleration of the masses _____________

b. the tension in the rope _____________

3. If the 8kg block from the system from problem 2 has a friction force of 25N acting on the 8kg, find:

a. the acceleration of the masses _____________

b. tension in the rope _____________

4. Two blocks (m1 = 7.0 kg and m2 = 13.0 kg) are connected by a rope over a pulley as shown.

a) What is the acceleration of m1? _____________

b) What is the acceleration of m2? _____________

c) What is the tension in the rope on m1? _____________

d) What is the tension in the rope on m2? _____________

e) The distance each objet will move in the first two seconds of motion if both objects start from rest. 											_____________

5. Two masses (m1 = 20.0 kg and m2 = 10.0 kg) are attached by a string as shown:

m2

a) Determine the acceleration of m2 assuming that the table

is frictionless. _____________

m1

b) Find the tension in the rope (no friction). _____________

c) Determine the acceleration there is a force of friction of 40N.

d) Find the tension on the rope (yes, now with friction). _____________

3. Three masses are attached (m1 = 19.0 kg, m2 = 11.0 kg, m3 = 5.0 kg) as shown.

m1

m2

Assuming no friction for, determine:

- What is the acceleration of the blocks?

_____________

m1

What is the tension in the string at point A?

_____________

c) What is the tension in the string at point B? _____________

4. Look at the diagram from question 3. If the force of friction on m2 is 35 N and the force of friction on m3 is 18 N, find their acceleration.		____________________